Models for estimating the cost of road public transit: a state of the art

Umberto Petruccelli , Pietro Vuono

Abstract

The estimate of the local public transport (LPT) standard cost is a topic of great interest in countries that subsidise these services with public resources, as it is essential to regulate the economic relations between the entrusting authority and the entrusted company but also to guarantee a fair distribution of the specific state resources among the competent local authorities. The subject has been addressed by some scholars who have produced estimation models supported by more or less complex mathematical relations, dependent on variables of a different nature.

This work illustrates the major results in the scientific literature and emphasises the interpretative capacity of each proposed model, assessed through the representativeness of the variables used and the calibration parameters. In particular, for each model, the variables on which it is based, the methodological approach and analytical form, the breadth and value of the reference sample are discussed, and its strengths and limitations are highlighted. The aim of this work is to systematise current knowledge on models for estimating the cost of LPT services by road, providing indications for future research with particular attention to the importance of the variables to be considered.

Keywords: Road public transport; Standard cost; Cost models; Production efficiency; Operating program; Driver productivity.

- The topic

In Italy, local public transport (LPT) is carried out by road for about 60% of mileage and is exercised under a State monopoly and delegated to the regions, which usually entrust services to external companies. In all European countries where LPT is subsidised, it is necessary to determine the reference cost to be used in public tenders [1].

Reference or standard cost is generally understood to be the cost of production per unit of product (vehicle⋅km or seater⋅km), calculated for a maximally efficient company, taking into account product specificities and actual operating conditions within a clear and well-defined territorial context [2], [3]. At the micro level, the reference cost is the basis for quantifying the subsidies to be paid to the companies operating the service. At the macro level, on the other hand, the same cost can be used to support an equitable distribution of resources between the regions of the same country, which must be based on actual needs in terms of quantity of service and unit costs to bear [4]. Actually, the principle of standard cost in the financing of public services formally entered the Italian legal system a long time ago to ensure a fair distribution and a more efficient use of public resources [5] even though, despite the regulatory efforts made to date, the objective of a fair distribution of state resources is still some way off [6]. Although there are precise instructions prepared by institutional sources, such as the Italian Transport Regulatory Authority (ART), the Italian Ministry of Infrastructure and Sustainable Mobility (MIMS) and the Regions, the greatest complexity in defining and developing models for calculating this cost lies in the lack of homogeneity in the data provided by the companies [3].

Over the last twenty years, the scientific literature on the estimating the cost per kilometre of LPT road services has been enriched by numerous models dependent on several variables, which generally include commercial speed (which effects the use time of drivers and vehicles) and the quantity of service produced (which reflects any economies and diseconomies of scale). Unfortunately, most existing models do not use variables describing even indirectly the characteristics of the service operating program (SOP) that significantly affect the efficiency of operators and rolling stock, and this limits the reliability of the results. In fact, the same number of hours of service supplied to the public (a direct consequence of the distance travelled and the commercial speed) can be realised using a different number of drivers and vehicles depending on the design of the lines and the timetable, with obvious consequences on the average annual mileage travelled by each driver and each vehicle.

The impact of the SOP on the cost per kilometre of the service is clarified by the example given in [7], to which we refer, which highlights the different use of drivers and buses on two identical extra-urban lines served by very different numbers of rides.

In the cost analysis worked out from the actual expense items, the influence of the SOP on the number of drivers and vehicles required is automatically taken into account, unlike in the estimation elaborated from the quantity and characteristics of the product made. In fact, estimation models are able to take into account the characteristics of the SOP only if they include variables that represent it, either directly or indirectly, through parameters indicative of driver efficiency, such as average annual mileage or the average daily ratio of driving hours to service hours. In particular, when using models that indirectly account for SOP through parameters indicative of driver and vehicle efficiency, it is possible to obtain reliable estimates of the service unit cost only if validated average values are attributed to these parameters, referring to services with the same characteristics as those under consideration. Considering that the cost of driving staff accounts for approximately 50% of the total, it is clear that using models that do not take into account the characteristics of the SOP leads to a cost estimate that is poorly representative of the specific service.

The models analysed in this work were initially taken from studies published by authors who have addressed the topic several times, with subsequent in-depth studies. Afterwards, the area of ​​investigation was significantly expanded to include other relevant, although less well-known, works. This led to the selection of 19 calculation models that constitute a sufficiently representative sample of the phenomenon studied.

In the following, 19 estimation models were examined (shown in Figure 1 in chronological order, by the authors' names) and were compared on the basis of the following features:

- the variables used and the reliability of the estimates;

- construction methodology and analytical form;

- the size and value of the data sample on which they were constructed.

Figura 1: Ordine cronologico dei modelli esaminati.

Figure 1: Chronological order of the models examined.

The prevalence in the literature of studies regarding the Italian context is a consequence of the growing importance that the standard cost has assumed in Italy over the last forty years in determining the economic compensation recognized by public contracting bodies to companies that operate TPL. This importance is also demonstrated by the issuing, by the Ministry of Infrastructure and Transport, of a specific Ministerial Decree (157/2018) which established methods and models for calculating the standard cost using data collected in recent years by the National Observatory for Local Public Transport Policies.

The originality of the work presented here lies in proposing a critical comparison (up to now lacking in the sector literature) between present models and in illustrating and valorising the variables influenced by the specific PdE that significantly affect the cost of TPL services and which, unfortunately, despite this, are neglected in most current models.

2. Model variables and estimate reliability

In the following, the models are classified into three categories on the basis of the variables used and are gathered in Tables 1, 2 and 3 and then discussed in the relevant subsections. In particular, we distinguish:

- Category 1: models in which the characteristics of the service operating program (SOP) are fully and comprehensively described by appropriate representative variables;

- Category 2: Models in which the characteristics of the SOP are only partially described through derived variables or discontinuous or dummy variables;

- Category 3: models that take no account the specificities of the SOP because they merely use variables representing general aspects of the service, such as the commercial speed and the quantity of service produced.

| CATEGORIA 1 - CATEGORY 1 |  |  |
| --- | --- | --- |
| Modello (autori) Model (authors) | Variabili utilizzate - Variables used |  |
|  | Variabili rappresentative del PdE Representative variables of SOP | Altre variabili Other variables |
| Petruccelli e Carleo (2017) [2] | HG.cond, TC, DS, FS, PF | vc, Q |
| Petruccelli e Racina (2019) [4] | X1, X2 |  |
| Simonelli et al. (2023) [8] | π, α, , Ndepotruns, wage | vc, Q |
| Legenda variabili - Variable legend: HG.cond = numero di ore di guida - number of driving hours; TC = durata temporale media delle corse - average running time of the rides; DS = densità del servizio – service density; FS = intensità del servizio – service intensity; PF = fattore dell’ora di punta del servizio - peak hour factor of the service; vc = velocità commerciale – commercial speed; Q = percorrenza annuale del servizio – annual mileage of the service. X1 = numero di corse offerte – number of supplied rides; X2 = tempo di percorrenza medio delle corse – average travel time of the rides; π = variabile che dipende dal contesto in cui viene svolto il servizio - variable depending from the context in which the service is performed; α = tasso standard di assenteismo dei conducenti ovvero del numero medio di giornate annue lavorate - parameter describing the standard rate of driver absenteeism, i.e. the average number of days worked per year;  = parametro che descrive le inevitabili inefficienze conseguenti alla corrispondenza tra i turni di lavoro e l’orario del servizio - parameter describing the inevitable inefficiencies resulting from the correspondence between work shifts and service hours;  = parametro che descrive l’incidenza dei percorsi a vuoto, cioè dello spostamento degli autobus da e per il deposito, all’inizio e alla fine del servizio - parameter describing the incidence of empty journeys, i.e. the movement of buses to and from the depot, at the beginning and end of the service; Ndepotruns = numero minimo di spostamenti “vuoti” ovvero da e per il deposito, all’inizio e alla fine del servizio - minimum number of 'empty' journeys i.e. to and from the depot, at the beginning and end of the service; wage = fattore correttivo che tiene conto della percentuale di autobus con età maggiore della vita utile standard assunta pari a 15 anni - correction factor taking into account the proportion of buses older than the assumed standard service life of 15 years. |  |  |

Tabella 1: Sintesi dei modelli di calcolo relativi alla categoria tipologica n°1.

Table 1: Summary of the calculation models for type category 1.

| CATEGORIA 2 – CATEGORY 2 |  |  |  |
| --- | --- | --- | --- |
| Modello (autori) Model (authors) | Variabili |  |  |
|  | Variabili rappresentative del PdE Representative variables of SOP |  | Altre variabili Other variables |
|  | Continue Continuous | Discontinue (o dummy) Discontinuous (or dummy) |  |
| Filippini et al. (2001) [9] |  | DEN2, REGIO | Q, n, T |
| Fraquelli et al. (2004) [10] |  | DINT, DMIX, | vc , Q, VC |
| Cirianni et al. (2012) [11] | HL.cond, Qcond |  | vc, Q, vutile |
| ANAV (2013) [12] | HG.cond |  | vc |
| ASSTRA (2013) [13] | HG.cond |  | vc |
| Di Gangi (2013) [14] | HL.cond |  | vc, Q, vutile |
| Veneto Region (2013) [15] |  | 5 categorie di servizio - 5 service categories | vc |
| Friuli V. G. Region (2013) [16] | HG.cond |  | vc |
| Avenali et al. (2018) [17] | HG.cond, vutile, av |  | vc, Q, Kmv, DKM1, DKM2 |
| Della Lucia et al. (2018) [3] | tu , t | α1, α2, α3 | h |
| Legenda variabili – Variable legend: DEN2 = fattore che dipende dal numero di fermate per km di rete servito - factor dependent on stops number per km of served network; REGIO = fattore dipendente dal contesto territoriale in cui viene svolto il servizio di trasporto - factor dependent on the territorial context in which the transport service is provided; Q = percorrenza annuale del servizio - annual mileage of service; n = lunghezza della rete - length of network; T = parametro che descrive le possibili fluttuazioni nel tempo - parameter describing possible fluctuations over time; DINT e DMIX = variabili dummy relative, rispettivamente, al servizio extra-urbano e misto - dummy variables relating to extra-urban and mixed service, respectively; vc = velocità commerciale - commercial speed; VC = fattore che descrive l’errore stocastico della funzione di costo - factor describing the stochastic error of the cost function; HL.cond = numero di ore lavorative del personale di guida - number of working hours of the driving staff; Qcond = media dei chilometri percorsi da ogni conducente in un anno - average number of kilometres driven by each driver per year; vutile = vita utile di ogni autobus - useful life of each bus; HG.cond = ore di guida nette di ogni conducente all’anno - number of driving hours for each driver in a year; Le 5 categorie di servizio, proposte dalla Regione Veneto, riguardano i seguenti ambiti territoriali: - urbano, in comuni con una popolazione inferiore a 150.000 abitanti; - urbano, in comuni con una popolazione maggiore o uguale a 150.000 abitanti; - extra-urbano, su percorsi pianeggianti o collinari (con quote fino a 600 m s.l.m.); - extra-urbano, su percorsi montani (con quote superiori a 600 m s.l.m.); - urbano, con mezzi a guida vincolata (metrotram). The 5 service categories, proposed by the Veneto Region, cover the following territorial areas: - urban, in municipalities with a population of less than 150,000 inhabitants; - urban, in municipalities with a population of 150,000 inhabitants or more; - extra-urban, on flat or hilly routes (with altitudes up to 600 m above sea level); - extra-urban, on mountain routes (with altitudes above 600 m above sea level); - urban, with guided vehicles (metrotram). av = età media dei veicoli - average age of the vehicles; Kmv = numero totale di bus.chilometro/anno per ogni veicolo - total annual mileage for each vehicle; DKM1 e DKM2 = variabili dummy che garantiscono la relazione tra gli altri costi e la quantità del servizio prodotto - dummy variables that support the relationship between the other costs and the quantity of service produced; tu= tempo di servizio corrispondente ad un “servizio base” – service time for a base service; t = volume di produzione espresso in termini di tempo di servizio erogato al pubblico – production volume expressed in terms of service time provided to the public; α1 = coefficiente rappresentativo del costo per ammortamenti, trazione e manutenzione connessi alle tipologie di bus e motorizzazione ed all’utilizzo - coefficient representing the cost for depreciation, traction and maintenance associated with the types of buses and engines and their use; α2 = coefficiente che descrive i fattori di costo indipendenti dalla velocità commerciale, legati agli assetti logistici ed alle scorte tecniche del parco mezzi - coefficient describing the cost factors independent from commercial speed, linked to logistical assets and technical stocks of the fleet; α3 = coefficiente che descrive il peso dei costi fissi di struttura - coefficient describing the weight of fixed structural costs; h = fattore di elasticità del costo rispetto alla quantità prodotta – coefficient of the cost elasticity with respect to the quantity produced.. |  |  |  |

Tabella 2: Sintesi dei modelli di calcolo relativi alla categoria tipologica n°2.

Table 2: Summary of calculation models for type category 2.

| CATEGORIA 3 – CATEGORY 3 |  |
| --- | --- |
| Modello (autori) Model (authors) | Variabili non rappresentative del PdE Variables not representative of SOP |
| Dalen e Gomez Lobo (2002) [18] | Ximt, Ym, μimt, εimt, Iimt |
| Cubukcu K. Mert (2006) [19] | Q, HDENSITY, HSTREET, HSLOPE |
| Abrate et al. (2014) [20] | QU, QI, QH, |
| Avenali et al. (2014, 2016) [5, 21] | vc, Q, A, DVC1, DVC2, DKM1, DKM2 |
| D.M. 157 (2018) [22] | vc ,Q, A, |
| Legenda variabili - Variable legend: Ximt = variabili specifiche dell’azienda in funzione del tempo - transport company-specific variables as a function of time; Ym = fattore che descrive le caratteristiche della rete stradale - factor describing the characteristics of the road network; μimt = inefficienza economica specifica di un’azienda - company-specific economic inefficiency; εimt = fattore rappresentativo dell’errore stocastico - factor representing stochastic error; Iimt = variabile dummy indicativa della tipologia di contratto - dummy variable indicative of the type of contract; Q = percorrenza annuale del servizio – annual mileage of the service; HDENSITY = densità di popolazione nell’area servita - population density in the served area; HSTREET = lunghezza media del tronco stradale compreso tra due intersezioni successive - average length of road section between two successive intersections; HSLOPE = percentuale di terreno pianeggiante - percentage of flat land; QU = quantità di servizio prodotta in ambito urbano - quantity of service produced in urban area; QI = quantità di servizio prodotta in ambito interurbano - quantity of service produced in extra-urban area; QH = quantità di servizio prodotta a noleggio - quantity of service produced for hire;  parametri che consentono la trasformazione Box-Cox - parameters allowing Box-Cox transformation; vc = velocità commerciale – commercial speed; A = ammodernamento del materiale rotabile per unità di servizio - rolling stock modernisation per service unit; DVC1, DVC2, DKM1, DKM2 = variabili dummy, utili a modellare l’andamento lineare a tratti della velocità commerciale e della dimensione del servizio - dummy variables, useful for modelling the linear trend of commercial speed and service size. |  |

Tabella 3: Sintesi dei modelli di calcolo relativi alla categoria tipologica n°3.

Table 3: Summary of the calculation models for type category 3.

2.1. Category 1

We selected three of the models falling into the first category, i.e. those in which the characteristics of the service operating program (SOP) are comprehensively and directly described through representative variables.

The model proposed by Petruccelli and Carleo (2017) [2] bases the cost estimation on a parameter called the 'Global Efficiency Indicator' (EG), which is able to synthetically represent the efficiency of the SOP and the company, as a product between the three following factors:

- average driver utilisation coefficient, consisting of the ratio between the average driving hours rendered by each driver and the number of hours worked, which measures the efficiency of the SOP and is a parameter external to the company, i.e. not controllable by the latter;

- cost of drivers on all staff cost ratio (Cdriv. /Cstaff), already used by ANAV [12] and ASSTRA [13], which indirectly measures company efficiency and represents an endogenous parameter of the company; in fact, a more efficient company achieves higher values of this ratio because it manages to contain the number of staff units other than drivers;

- all staff cost on the total service cost ratio (Cstaff /C), which is also already present in the ANAV and ASSTRA models (see subsection 2.2), is only partially an endogenous parameter as it partly describes the efficiency of the company and partly that of the SOP.

The authors defined, on a regression basis, an analytical relationship that makes it possible to determine the overall efficiency indicator as a function of six descriptive variables of the SOP, which are: the annual mileage of the service, the average commercial speed, the average time of the rides, the density and intensity of the service, and the peak hour factor of the service. Once the value of EG is known and the annual service hours are determined, the cost estimate is immediate. This model has the advantage of the simplicity in the calculation and methodological approach as well as the use of only inputs known also outside the company.

Only a few of the cost models proposed in the literature include variables representative of driver productivity. The latter is a factor that is exogenous to the production company as long as it depends almost exclusively on the SOP but it is difficult to know outside the production company because it can only be measured downstream in the construction of the work shifts. The model developed by Petruccelli and Racina [4] brings out this important cost driver by comparing a supply scheme with all-direct links (direct-link) and one with all-integrated links i.e. comb type (feeder-trunk), serving the same area and characterised by the same overall mileage. As the authors demonstrate with the application to a real case, these two schemes reach very different productivity per driver. In particular, the latter proves to be much more efficient than the former because it consists of shorter links served by a higher number of rides and therefore allows for a higher use of drivers during the work-shift. This results in a greater mileage travelled by each driver and therefore a lower cost per kilometre. This shows how an estimate of the cost per kilometre based on a value of the mileage travelled per driver taken as an average from a sample of companies leads to an incorrect result because it is independent of the specific SOP implemented.

To allow the number of drivers to be estimated with reference to the specific SOP, without knowing the work-shifts, the authors constructed two regression models, one for each of the two supply schemes considered, based on two descriptive variables of the SOP, which were found to be the most significant among several analysed, namely the 'number of rides supplied' and the 'average trip time of the rides'. It is therefore possible to use these relations to estimate the number of drivers actually required for the specific SOP and to use this value in other standard cost estimation models such as the ANAV or ASSTRA models by substituting in the latter, for the average mileage travelled by each driver obtained as an average sample value, the value estimated on the actual SOP.

With the aim of supporting LPT companies in improving performance, Simonelli et al. (2023) [8] followed a different approach for estimating the operating cost that led to a model based on context and service variables. The total cost is calculated as the sum of the main cost items represented by drivers, marketing and administrative staff, depreciation and other fixed costs, depot maintenance and staff, and traction. This model, taking its cue from a number of previous studies, determines the average number of annual driving hours achieved by each driver as the product between a standard value and a variable dependent on the context in which the service is carried out. The first factor is a function of the maximum number of annual driving hours permitted by regulations and the average number of annual days worked. The second factor, on the other hand, depends on two parameters, one of which describes the inevitable inefficiencies resulting from the correspondence between the work-shifts and the service timetable and the other describes the incidence of empty runs, i.e. the movement of buses to and from the depot, at the beginning and end of the service. In addition to the different approach in estimating the number of buses and drivers, the model differs from others in the literature since it calculates the depreciation cost of each bus. This is also a function of a correction factor that takes into account the percentage of buses with an age greater than the assumed standard service life of 15 years. The results showed the model's ability to estimate a cost value very close to the real one as well as correctly assessing the inefficiencies of a company according to the specific characteristics of the service and the context in which it is carried out.

2.2. Category 2

Among the models belonging to the second category, a significant contribution to standard cost estimation was made by the model of ANAV (Italian Associazione Nazionale Autotrasporto Viaggiatori – Italian National Travellers' Road Transport Association) and University of Rome "La Sapienza" [12]. This method indirectly takes into account the SOP, through the average number of driving hours rendered by each driver (HD.driv) and considers production efficiency parameters represented by ratio of the driving staff cost on that of the entire staff (Cdriv /Cstaff) and the ratio of the entire staff cost on that of the total service (Cstaff /C). In order to obtain a minimum and a maximum cost, all the parameters used are assumed to have two extreme average values, taken from a sample of transport companies associated with ANAV, distinguished on the basis of the type of service produced, whether urban or extra-urban. The procedure is based on the estimate of the cost per kilometre of the drivers, from which, given the ratios (Cdriv /Cstaff) and (Cstaff /C) defined above, the total cost per kilometre is derived; to the latter must be added company profit and IRAP (Italian Regional tax on productive activities). The cost per kilometre of the drivers is obtained by dividing the annual cost of each unit by the average mileage travelled by each one. The latter is derived from the product of the average annual working days of each driver, for the average daily driving hours performed by each one and the average commercial speed of the service. The parameter that has the greatest weight and is closely linked to the characteristics of the SOP is therefore the efficiency of the drivers, expressed by the number of daily driving hours. For this parameter, the authors provide average values (shown in table 5) measured on the sample selected by ANAV.

Another methodology of application interest is defined by the ASSTRA (Italian Transport Association) model [13], which provides a cost splitting between the various business processes and evaluates the efficiency, not only of the individual production factor, but of the entire process area. Therefore, four main areas are identified with their corresponding cost items shown in Table 4.

| Area di costo Cost area | Voce di costo Cost item |
| --- | --- |
| Esercizio Operation | Personale di guida, personale di movimento, personale di deposito, carburante, lubrificante, pneumatici, ammortamento dell’autobus e delle rimesse, Information and technology, costi fissi connessi gli autobus, altri costi. Driving staff, movement staff, depot staff, fuel, lubricant, tyres, depreciation of bus and depots, information and technology, bus-related fixed costs, other costs. |
| Manutenzione Maintenance | Personale di manutenzione, altri costi di manutenzione. Maintenance staff, other maintenance costs. |
| Marketing, comunicazione e qualità Marketing, communication and quality | Costo del personale commerciale; Altri costi marketing e comunicazione. Cost of commercial staff; Other marketing and communication costs. |
| Amministrativa Administrative | Costo del personale amministrativo; Altri costi (consumi, godimento beni di terzi, oneri diversi di gestione, altri ammortamenti). Cost of administrative staff; Other costs (consumption, leases and rentals, miscellaneous operating expenses, other depreciation). |

Tabella 4: Aree e voci di costo nel modello ASSTRA [13].

Table 4: Areas and cost items in the ASSTRA model [13].

As in the ANAV model, the final unit cost is increased by the return on capital (business profit) and IRAP. The operating procedure is the same as in the ANAV model, the difference being that slightly different values are given for the parameters derived from the sample of affiliated companies, which are compared in Table 5.

It should be noted that, although the ANAV and ASSTRA models were developed around ten years ago, they represent a reference in the standard cost estimation methodology. In particular, the former distinguishing itself by a macro-analytical approach that relates the cost of the service to productivity parameters, while the latter by a micro-analytical approach that analyses all the cost items of the production process of a transport company, which allows the evolution of each individual factor of production to be assessed. A common feature is the evaluation of two scenarios, the best and the worst, based on the values assumed by the parameters considered. They have many similarities and are conceptually unimpeachable, but their limitation is that they use parameter values taken from a sample of member companies, not validated outside the associations themselves. Moreover, these parameters are evaluated by distinguishing only between urban and extra-urban services without taking into account other characteristics of the offer, such as the extension of the lines and the frequency of rides, which significantly influence the unit cost.

The model proposed by the Friuli-Venezia Giulia (F.V.G.) Region (Italy) [16], on the other hand, aims to provide a confirmation of the congruity of the current economic compensation (which, in this territorial context, allows a satisfactory business profit), as well as to cater a reference for the evaluation of current levels of business efficiency. The operating procedure is the one introduced by ANAV, with inputs contextualised to the regional territory (see Table 5). The estimate of the standard economic compensation, conducted using the model in question, provided results of acceptable approximation at the plan scale. In particular, the standard economic compensation estimated with the tool in question is greater than that obtained by rigorously applying the ANAV method since, in the calculation procedure, the standard revenues (equal to 35% of the costs) were not considered, but rather the actual revenues, which are significantly lower. In order to manage the large quantity of data and to carry out accurate economic evaluations, the authors made use of a 'technical-economic simulator' that reconstructs the company's profit and loss accounts on the basis of technical, economic and production parameters, allowing comparative analyses to be developed on homogeneous indicators and simulations of the impact of changes in efficiency and productivity.

| Parametro | Modello ANAV ANAV model |  |  |  | Modello ASSTRA ASSTRA model |  |  |  | Modello F.V.G F.V.G. model |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Scenario peggiore Worst case |  | Scenario migliore Best case |  | Scenario peggiore Worst case |  | Scenario migliore Best case |  |  |  |
|  | Urbano Urban | Extraurb Extraurban | Urbano Urban | Extraurb Extraurban | Urbano Urban | Extraurb Extraurb. | Urbano Urban | Extraurb Extraurban | Urbano Urban | Extraurb Extraurb. |
| Giornate lavorative annue Annual working days | 255 | 255 | 255 | 255 | 250 | 250 | 250 | 250 | 255 | 255 |
| Velocità commerciale minima (km/h) Minimum commercial speed (km/h) | 17 | 30 | 17 | 30 | 15 | 28 | 16,5 | 30 | 17,27 | 39,05 |
| Ore di guida al giorno Driving hours per day | 5,5 | 4,5 | 5,75 | 4,75 | 5,25 | 4,50 | 5,25 | 4,50 | 5,50 | 4,50 |
| Costo annuo di un conducente (Euro) Annual cost of a driver (Euro) | 42.000 | 42.000 | 42.000 | 42.000 | 40.552 | 40.552 | 40.552 | 40.552 | 43.470 | 43.470 |
| Costo conducenti / costo lavoro totale (Ccond/Cpers) Driver cost / total labour cost (Cdriv /Cstaff)) | 75% | 75% | 80% | 80% | 73% | 66% | 76% | 76% | 77% | 77% |
| Costo del lavoro / costo totale (Cpers/C) Labour cost on total cost (Cstaff /C) | 50% | 50% | 52% | 52% | 60,6% | 52,1% | 54,8% | 43,6% | 52% | 52% |
| IRAP da coprire Local taxes | 0,09 | 0,06 | 0,08 | 0,06 | 0,11 | 0,08 | 0,10 | 0,06 | - | - |

Tabella 5: Raffronto fra i valori dei parametri utilizzati nei modelli ANAV [12], ASSTRA [13] e F.V.G [16].

Table 5: Comparison of parameter values used in ANAV [12], ASSTRA [13] and F.V.G [16] models.

Filippini et al. (2001) [9] related cost to certain factors such as labour, capital and fuel to which they added two dummy variables describing the density of stops and the orography of the territory. It should be pointed out, however, that the variables considered, although representative of certain characteristics of the SOP, are in no way affected by the time spent by drivers at the terminus, during their work-shift, waiting for the time of the return journey and are therefore not representative of the influence of this important aspect of the SOP on the annual mileage produced by drivers. The results obtained consist of unit production costs, as a function of the values assumed by the two dummy variables and the production in bus/km, which can be used for companies operating in the road LPT sector as an evaluation parameter in order to limit the distortions between the costs defined by the regulator and those sustained by the regulated subject, as specified by the authors.

Fraquelli et al. (2004) [10], on the other hand, proposed a cost function comprising variables concerning labour, fuel and material and service charges as well as two service-specific discontinuous variables for extra-urban and mixed companies, precisely because they wanted to verify the presence of possible economies of scope deriving from a mixed production combining urban and extra-urban transit. The model in question also includes the average commercial speed to take into account the different traffic and geographical conditions of the area in which the service is supplied as well as a factor describing the overall stochastic error. The number of drivers, although it does not explicitly appear in the proposed model, influences the variable representing labour costs. The analysis shows the possibility of obtaining economies of scale through mergers between LPT companies operating on an integrated urban and extra-urban network.

Di Gangi (2013) [14], on the other hand, proposed a slightly different methodology that consists in defining, through a quantitative analysis, the resources useful for providing a given transport service once the existing demand and the fare structure are known. In particular, the model consists of the following steps aimed to quantify:

- the activities carried out to provide the service on the line;

- the type of resources used for the development of individual activities;

- the acquisition cost of these resources;

- the resources deployed on the line;

- direct line costs by activity.

From an analytical point of view, the formulation to determine the cost depends on three attribute vectors describing, the first the service produced, the overall size of the service and the technological aspects, the second the time aspects related to the service produced, the third the rigid cost component and the additive component that varies with the amount of service produced. At this point, the total operating cost is determined through the sum of the cost of traction, maintenance, staff, plant and vehicles, and overheads. The unit cost of the drivers was determined on the basis of the average commercial speed of the service and the average daily driving hours rendered by each driver deduced from average values for a given sample of companies. This model proposes a methodology for estimating and evaluating costs, where each SOP can be described through various parameters such as line structure, location of stops, frequencies, travel times between adjacent stops, timetables, vehicle capacity, etc. This defines a useful tool that allows the authorities to determine in advance the economic resources to be provided to enable the planned services to be carried out in an optimal manner.

The methodology proposed by Cirianni et al. (2012) [11] provides for the estimation of the drivers needed in local public transport services by deriving it from the average annual mileage produced by each one. The latter value is estimated from a sample of companies on a production of 24,000 km/driver.

The model proposed by the Veneto Region (Italy) [15] determines the standard cost as the summation of eighteen items whose reference values were obtained from statistical processing of both directly collected data and their subsequent transpositions. In particular, the procedure adopted quantifies a series of specific kilometric costs for each homogeneous category of service. Within the same category, however, the services differ in terms of the mileage travelled, the size of the operating company, the specific area in which the services are carried out, industrial policies and the extent of investments. In this case, the process of constructing the standard cost per kilometre provided for five different types of service, four of which are defined according to operating conditions (environmental conditions) and the fifth category is distinguished by the technology used (metrotram). The service categories concern:

- urban in municipalities with a population of less than 150,000 inhabitants;

- urban in municipalities with a population of 150,000 or more;

- extra-urban on flat or hilly routes (up to 600 m above sea level);

- extra-urban, on mountain routes (at altitudes above 600 m above sea level);

- urban, with captive guided vehicles (metrotram).

Overall, we turned out that about half of the proposed models use commercial speed as the main input, in contrast to the model of Petruccelli and Racina (2019) [4] (described above) and that of Della Lucia et al. (2018) [3]. The latter, which calculates the total cost of production only as a function of the service time and other coefficients, has the advantage of speed use as it places the standard cost as a function of service time only. This model is based on the following assumptions:

- the presence of variable costs (such as traction and driving personnel costs) related only to service time;

- the variability of fixed costs (such as those related to structural staff, logistics and the geography in which the service is produced), in relation to the size of production;

- the presence of non-linearly varying economies of scale.

The authors have constructed an exponential formula to determine the total cost of production, in which the variable is the service time referred to a basic service in order to neutralise the effects of reduced productions for which there is a strong dispersion of cost values. There are also two parameters in the formula: the first, that is the cost to be supported to provide an extra-urban service considered as a basic service, and the second, by an adimensional coefficient describing the effect of the specific structural characteristics of the service independent of the commercial speed, as well as of the supply spread over the territory and of organisational polycentrism. The latter is the product of three other parameters, one of which represents the cost factors relating to depreciation, traction and maintenance costs associated with the types of buses and their use in the various services; another describes the cost factors independent of commercial speed, linked to the logistical set-up and technical stocks of the fleet and the last considers the weight of fixed structural costs.

In 2018, Avenali et al. [17] proposed a hybrid model that thus exploits the benefits of both statistical and engineering approaches (see section 3). The model makes use of the bottom-up approach for the estimation of the burden of driving staff and rolling stock while it makes use of the top-down approach for the estimation of the remaining items. In particular, the cost of driving staff depends on the average driving hours per year produced by drivers deducted from a sample of companies. The capital cost is based on the minimum rate of return on net investment and is a function of the average age of vehicles. The authors note the existence of a non-linear link between "other costs" (i.e. that part of the total cost that is determined through the top-down approach) and commercial speed as well as the presence of weak economies of scale. In particular, economies of scale decrease as the size of the service increases and turn into diseconomies when the threshold of 6 million of bus-kilometres is exceeded. The authors argue that, in order to improve efficiency, diseconomies of scale should not be taken into account in the calculation of the standard unit cost because then regions with a large transport service would have an incentive to pursue efficiency by reducing the average size of the lots to be tendered. Therefore, the analytical relation allowing the calculation of the other costs must only take into account economies of scale up to the threshold of 6 million of bus⋅km.

2.3. Category 3

Finally, the main models that do not take the characteristics of SOP into account are recalled. In chronological order, the first is that of Dalen and Gomez Lobo (2002) [18] which uses an approach different from the others, i.e. instead of studying production costs, it analyses the economic efficiency of certain transport companies over time by means of a specially defined parameter. The relationship constructed also depends on parameters relating to the company and the characteristics of the road network as well as a dummy variable indicative of the type of contract and a term representing the stochastic error. The aim of the study was to find out how the performance of a company is influenced by the different contract type. The analysis of the results, in contradiction to some studies reported in the literature but consistent with the observed empirical phenomenon concerning Norwegian companies, revealed the absence of a relationship between company economic efficiency and switching to an incentive contract.

In 2006, Cubukcu [19] proposed a model whose purpose is to test whether the physical and geographical characteristics of a spatial context are able to influence the cost of bus transit. The results show that the total cost decreases with increasing population density in the area served, the average length of the road section between two successive intersections and the percentage of flat route. The thresholds above which the unit cost increases are also identified in the various cases.

Abrate et al. (2014) [20], on the other hand, have defined a formula to determine the cost of production in the long run, distinguishing the quantity of service produced and the operational context (urban, extra-urban and chartering), according to the cost of production factors and in particular, labour in each sector, capital and fuel. The purpose of the work is to search for possible economies resulting from mergers between companies or integrations between services of different types, i.e. urban, extra-urban and chartering. Such economies were found only in the case of mergers between multi-service companies with a relatively low level of production. In addition, it emerged that extra-urban services are those that can most easily be combined with urban services and chartering services, but if the company is characterised by small size and operates in a small area, in order to increase its efficiency, it should pursue economies of purpose as well as economies of scale and, in particular, focus more on the integration of extra-urban services with urban ones than with chartering ones. Thus, small integrated companies operating in the same territorial context show economies of scale and scope, while large integrated companies show clear diseconomies of scale.

In 2014, Avenali et al. [5] developed a model able to estimate the unit cost per kilometre of a road LPT service as a function of commercial speed, the amount of service produced and the level of modernisation of the rolling stock, as well as some dummy variables, useful for modelling the linear trend of the cost as commercial speed and the size of the service change. The results showed how, as commercial speed increases, the standard cost per kilometre decreases at decreasing marginal rates. With respect to the size of the service, on the other hand, the cost function initially presents a decreasing trend and then grows, therefore, the presence of economies of scale is detected up to a threshold value, which the authors indicate is equal to 4 million bus.km/year, beyond which, on the contrary, diseconomies are revealed. The proposed model, by identifying a functional link between unit cost and variables that are in part uncontrollable by operators, such as commercial speed and kilometres supplied, provides an incentive for regions and local authorities both to improve the contextual conditions in which local public transit services are supplied and to identify the optimal size of the basins, thus providing important policy indications.

With reference to the same data base and the same variables, in 2016, the same authors optimised the model and, by means of a statistical analysis and consistent with certain observations deduced from interviews administered to company executives, they formulated analytical reports for three types of models [21]. The results show, how the incidence of commercial speed on the unit cost decreases as it increases, and that economies of scale are weak and tend to become diseconomies when the size of the service reaches a threshold of 4 million bus.km per year.

Special attention, as it plays the role of a standard in Italy, deserves the standard cost model for bus-line services introduced by Ministerial Decree 157/2018 of the Italian Ministry of Infrastructure and Transport [22]. The tool, which is constructed by regression on the data collected by the National Observatory for Local Public Transport Policies, makes the cost depend on three variables, which are commercial speed, quantity of service produced and rolling stock modernisation per service unit. For a better understanding of the trend of the values calculated through this model, it is very useful to refer to the results of a recent analysis conducted by [23]. Here it is shown that the cost calculated according to the DM in question decreases linearly as the commercial speed increases with three different inclinations of the representative straight lines, according to the speed range considered and as the quantity of service produced varies. Specifically, it is verified that, in the interval between 12 and 17 km/h, the standard cost shows the greatest sensitivity to commercial speed by realising reductions in the former of the order of 15% as a consequence of an increase in the latter of approximately the same magnitude. Consequently, especially in urban areas, increasing bus speeds by reducing road congestion achieves considerable savings in operating costs as well as considerable environmental benefits as a result of smooth vehicle flow. For speeds above 17 km/h, the cost trend drops significantly and further above 32 km/h (figure 2.a).The main aspect concerns, consistently with Avenali et al. 2014 [5], the U-shaped trend of the unit cost curve as the quantity produced varies, which reaches a minimum at 4 million bus.km and remains constant for travel values greater than 10 million bus.km (figure 2.b). It follows that, if a contracting authority divides the services into lots of 4 million bus.km each, it will obtain a minimum basic tender cost; if, on the other hand, it merges the services into lots of size equal to or greater than 10 million bus.km, it will be forced to associate with them a higher basic tender cost of between 14% and 70% depending on the commercial speed. Therefore, some perplexity, in terms of reliability, arises from the trend in costs as the quantity varies, although this is consistent with the findings of some studies [20, 24]. However, it is evident that the three variables on which the cost function is constructed do not take into account the specificities of the SOP implemented, which on the other hand is capable of significantly influencing the number of drivers required and the mileage travelled by them. This represents a major limitation to the reliability of the estimate made with the model of Ministerial Decree 157/2018.

Figure 2.a e 2.b: Andamento del costo standard per il TPL su gomma, secondo il modello proposto dal D.M. 157/2018 al variare della velocità commerciale e delle percorrenze complessive del servizio.

Figures 2.a and 2.b: Trend of the standard cost for road local public transport, according to the model proposed by Ministerial Decree 157/2018 with variations in commercial speed and overall service mileage.

3.   Methodology and analytical form of the model

Cost models can also be distinguished, based on the type of approach, into the three types described below [2].

- Top-down or statistical: it defines the cost on the basis of functional links and statistical analyses deduced from accounting and operational data provided by transport companies operating in the territory; it follows that, if the efficiency of the companies in the sample is not ascertained, the estimates may be influenced by the specificity of the companies. Of course, if company data is not available, the model cannot be used.

- Bottom-up or Engineering: defines cost analytically from production and technology standards and requires accurate analysis of costs incurred in specific business areas. The application of such models, therefore, presupposes a mastery of production processes, individual factors and their respective costs, and presents the risk of determining an 'ideal' cost value, i.e. unrelated to the actual operating conditions of the production context.

- Hybrid: it presents intermediate characteristics between the two methodologies just mentioned and can therefore, if correctly set up, reduce the criticalities associated with each of them.

Approximately 70% of the authors here cited favoured the statistical approach in constructing their models, which overcomes the need for a clear and detailed knowledge of the production processes, but entails an estimation that is significantly influenced by the representativeness of the data in the selected sample and obviously does not provide indications of the functional links between the cost and the model variables. The models proposed by Filippini et al. (2001), Dalen and Gomez Lobo (2002), Fraquelli et al. (2004), Cubukcu (2006), ANAV (2013), Abrate et al. (2014), Avenali et al. (2014, 2016), Petruccelli and Carleo (2017), Della Lucia et al. (2018), Ministerial Decree no.157 (2018) and Petruccelli and Racina (2019) belong to the top-down category. The Friuli-Venezia Giulia Region (2013), on the other hand, adopted a simplified approach which can nevertheless be considered as belonging to the category of statistical models. Instead, the model proposed by Cirianni et al. (2012), the ASSTRA (2013), the Veneto Region (2013), Di Gangi (2013) and Simonelli et al. (2023) fall into the bottom-up category. Only in one case, (Avenali et al., 2018), as anticipated, an advanced numerical strategy was adopted represented by a hybrid model, able to effectively combine the bottom-up approach to determine the cost rate related to the driving staff and the vehicle fleet, flanked by a top-down approach to estimate the remaining cost items.

As far as the analytical form is concerned, on the other hand, it was found for the case history examined that most of the models employed a fairly well-established mathematical procedure, represented by a cost function with a trans-logarithmic form. Filippini et al. (2001), Dalen and Gomez Lobo (2002), Fraquelli et al. (2004), Cubukcu (2006) and Abrate et al. (2014) adopted this formulation. In particular, Cubukcu (2006) employed a trans-logarithmic function previously developed by other authors and represented, in this case, in an approximate way, by means of second-order arrested Taylor development, while Abrate et al. (2014) defined a formula to determine the standard cost on the basis of the relationship introduced in 1992 by Pulley and Braunstein [25] called the econometric model of the composite cost function. Other authors, conversely, have employed a cost function deduced through statistical procedures based on multiple linear regression. This is the case of Petruccelli with Carleo (2017) and Racina (2019) as well as of the methodology dictated in Italian Ministerial Decree no.157/2018. The latter uses linear regression to construct the cost model for bus lines and that for regional railways while, for tramways and metros, for which a sufficiently extensive database was not available, it indicated a well-defined calculation procedure for which the values of many parameters were also provided. The method of least squares was chosen by Avenali et al. in 2014 and 2016, and a similar mathematical approach was also adopted in the 2018 hybrid model limited to the calculation of part of the cost items, as well as by Della Lucia et al. (2018), to define an exponential relationship. There are also models that determine the cost of the LPT service by analysing the individual items that make up the entire production process. Such an approach has been applied by the model of Cirianni et al. (2012), ANAV (2013), ASSTRA (2013), Veneto Region (2013), Di Gangi (2013) and by Simonelli et al. (2023).

In Tables 6 and 7, models are grouped according to approach type and analytical form.

| APPROCCIO - APPROACH | AUTORI - AUTHORS |
| --- | --- |
| “Top Down” o Statistico 'Top Down” or Statistical | Filippini et al., Dalen e Gomez Lobo, Fraquelli et al., Cubukcu, ANAV – Università di Roma “La Sapienza”, Regione Friuli-Venezia Giulia, Abrate et al., Avenali et al. (2014 e 2016), Petruccelli e Carleo, Della Lucia et al., D.M. 157/2018, Petruccelli e Racina. |
| “Bottom Up” o Ingegneristico "Bottom Up” or Engineering | Cirianni et al., Regione Veneto, ASSTRA, Di Gangi, Simonelli et al. |
| Ibrido - Hybrid | Avenali et al. (2018) |

Tabella 6: Modelli di costo suddivisi per tipo di approccio.

Table 6: Cost models by type of approach.

| FORMA ANALITICA ANALYTICAL FORM | AUTORI AUTHORS |
| --- | --- |
| Trans-logaritmica - Metodo dei minimi quadrati generalizzati (GLS) Trans-logarithmic - Generalised Least Squares Method (GLS) | Filippini et al. |
| Trans-logaritmica - Regressione iterata apparentemente non correlata Trans-logarithmic - Iterated Seemingly Unrelated Regression (ISUR) | Fraquelli et al. |
| Trans-logaritmica (generica) Trans-logarithmic (generic form) | Dalen e Gomez Lobo, Cubukcu |
| Trans-logaritmica generalizzata (GT) - Modello econometrico della funzione di costo composito Generalized trans-logarithmic (GT) - Composite Cost Function Econometric Model | Abrate et al. |
| Modello Econometrico, Relazione Lineare - Metodo dei minimi quadrati Econometric Model, Linear Relation - Least Squares Method (OLS) | Avenali et al. (2014, 2016, 2018) |
| Relazione Esponenziale - Metodo dei minimi quadrati Exponential Relation - Method of least squares | Della Lucia et al. |
| Metodo statistico della regressione multipla lineare Linear multiple regression statistical method | Petruccelli e Carleo, D.M. n.157/2018, Petruccelli e Racina |
| Metodo “semplificato” ‘Simplified’ method | Friuli-Venezia Giulia Region |
| Analitico (Generico) Analytical (Generic) | Cirianni et al., ANAV, Di Gangi, Regione Veneto, ASSTRA, Simonelli et al. |

Tabella 7: Modelli di costo suddivisi per forma analitica.

Table 7: Cost models by analytical form.

4. Size and validity of the data sample

The size and robustness of the database used in the building and validation of a model has a considerable weight on the reliability of the results it is able to produce. However, it should be considered that the ability of the database to reflect the cost of LPT services is also conditioned by the time to which the values contained in it refer, which are certainly affected by the different rules that have regulated the valuation of standard costs and compensations over time, as well as the mechanisms for entrusting services. Undoubtedly, the aspect that most reinforces the reliability of the database and of the model constructed from it concerns the quality of the data demonstrated through validation by an independent certifying body. In fact, while the values obtained directly from a specific service are certain insofar as they are actually found in reality, even if they are not necessarily representative of a production of maximum efficiency, those deduced from a sample of companies may be considerably inhomogeneous if they present different shapes and sizes or produce services that differ in terms of characteristics and size or operating context. While in a sufficiently large sample it is possible to group the different services according to travel distance, commercial speed, urban or extra-urban area of implementation or according to other characteristics, it is generally difficult, in a small sample, to make distinctions on the basis of the specifics of the service operating program (SOP) or operating context. Therefore, the model built on such a sample is generally unable to take into account the SOP and consequently its application will produce a sensitive under- or over-estimation. On the basis of these considerations, Petruccelli and Racina (2021) [26] have distinguished, into high, medium and low, the level of reliability of the variables used in the cost models, on the basis of how the relative values are derived from time to time, i.e. whether based on the specific service or on a sample of companies, highlighting those variables for which, more than for others, the assumption of average sample values affects the reliability of the estimates made with the model. From this it emerges that the variables, whose values require a close reference to the specific service, are the most important ones, such as the "annual driving hours of a driver", from which the number of drivers required for the service is directly derived. The established methodology for estimating the cost of road LPT makes use of parameters, either referred to the specific service or representative of sample average values since they are not directly detectable. The parameters of the first type are usually the annual service hours rendered, the commercial speed and the annual mileage, while the second type includes the annual driving hours per driver and the ratio between the cost of drivers and that of staff as a whole. The resulting estimate will be all the more accurate and reliable the fewer and less weighty the parameters of the second type are used. It can therefore be understood that the determination of the variables, on which the standard cost depends, should be based on values drawn from services actually performed.

With regard to the spectrum investigated, the authors who made use of a larger sample were Dalen and Gomez Lobo (2002) with 1136 observations taken from Norwegian transport companies between 1987 and 1997 and Cubukcu (2006) with 1061 observations, taken between 1996 and 2002, relating to 264 transport companies in the USA, which were combined with geographical and physical data and processed using GIS. More contained are the samples that supported the construction of the model by Filippini et al. (2001) (170 observations, spanning from 1991 to 1995, relating to 34 Swiss transport companies), that of Abrate et al. (2014), (147 surveys collected from 2008 to 2012, relating to 47 Italian LPT companies) and also that of Fraquelli et al. (2004), (135 observations relating to 45 municipal public transport companies belonging to Federtrasporti - an Italian association, operating in the period between 1996 and 1998). As can be seen, the samples used by the aforementioned authors, although numerically representative, sometimes substantial, and spanning a fairly broad average period (4 - 5 years), are nonetheless dated over twenty years ago.

On somewhat more recent data are based the elaborations carried out by Avenali et al. in 2014, 2016 and 2018, which made reference to a sample collected in 2011, consisting of 45 Italian companies with a production of approximately 500 million bus.km/year, and ASSTRA, which made use of data collected in 2012 through questionnaires administered to its member companies. Similarly, the Veneto Region constructed its own analysis sample by collecting the cost data of all the companies operating on its territory, entrusted with LPT in the years 2010 and 2011, and integrating them with information taken from the 'National Statistical Programme - LPT Survey', while the Friuli Region carried out a numerical simulation with reference to the year 2009.

The sample used by Della Lucia et al. (2018) for the construction of their model, on the other hand, was provided by ten Italian companies of small and medium size and operating in both urban and extra-urban areas and, despite the relatively small size of the database, the results obtained demonstrate a fair ability of the model to reproduce real costs.

However, as can be seen, most of the models analysed used an unvalidated sample of data, the reliability of which was assumed a priori and this, although it does not limit the validity of the methodology and the tool constructed, does not guarantee its good calibration and thus the correspondence to reality of the results that can be calculated.

Petruccelli and Carleo (2017), on the other hand, considered a sample that, although numerically very small, nevertheless reports costs that are certainly real and of maximum efficiency as they only refer to services entrusted with an insignificant bidding discount, following a public tender. This ensures that the cost put out to tender is indeed the minimum cost and therefore the cost of maximum efficiency.

Certainly, the model contained in DM.157/2018 was built on the basis of the broadest and most reliable source that is the data collected in Italy by the Italian National Observatory for Local Public Transport Policies. Therefore, the estimation tool provided by the MIT, in addition to having standard effectiveness, is to be considered the most reliable in terms of calibration as well as the most representative of the Italian situation, although it has important limitations in its ability to reproduce reality due to the lack of any representative variable of the characteristics of the SOP and therefore of the productivity of drivers (as discussed above). By the way, a strong similarity was found between the model defined by DM.157/2018 and the one proposed by Avenali in 2014, the only differences being in the values of some assumed parameters.

Finally, Simonelli et al. (2023) made use of General Transit Feed Specification (GTFS) traffic data produced by Google, but the model was calibrated with reference to a specific real-life case related to an 11.4 million bus.km service offered in the city of Naples (IT) in 2018.

5. Final remarks

This study works out a state of the art on the estimation of the standard cost for a bus LPT service by critically analysing the most significant models in the scientific literature.

As is well known, while cost analysis results from the sum of the various items of expenditure for the purchase of production factors, estimation involves tracing the cost of production from the quantity and characteristics of the product and the production process. Therefore, estimation is the only way to go when the details of the expenditure items of the specific production are not known or when a cost independent from the efficiency of the company performing the service, i.e. a standardised value, is required. Estimation needs the use of a method or model constructed for the specific sector under investigation and calibrated to the reality that is to be reproduced.

The ability of a model to reproduce a real phenomenon is a function of two aspects: the number and representativeness of the variables it includes and the good calibration of the constants. The first aspect cannot disregard a detailed analysis of the phenomenon under consideration and the relevance of the variables influencing it. The second requires the availability of a mass of data representative of the phenomenon itself and the variables connected to it, suitably validated and referring to a sufficiently extended temporal or spatial range.

The variables that have a bearing on the standard cost of local public transport by road are the service operating program (SOP) (the product), the operating company (the producer) as well as the road, and the contractual relationship (the operating environment). The most commonly used variables are the commercial speed and the annual mileage of the contracted service. The first variable is particularly affected by the average distance between stops (characteristic of the SOP product) as well as by the functional standards and traffic conditions of the infrastructure (operating context) and measures the hourly use of driving staff and vehicles for the realisation of the product; it is sometimes replaced by the service time rendered to the public, which is a direct measure of the time of utilisation of the factors of production in question. The second variable is indicative of economies and diseconomies of scale related to the total mileage travelled (quantity of the product) and the operating environment that may affect the efficiency of production. Often the models also include one or more variables indicative of the fixed costs related to rolling stock, including the depreciation quota that may fall on the service operator.

It is evident that there are no variables capable of representing one of the three aspects (product, producer and operating context) separately, but each one describes several aspects simultaneously in a partial manner. Therefore, in the construction of the cost model, the choice of variables is very important because the use of variables that are not sufficiently representative or too few in number risks making the instrument not effective enough in reproducing the phenomenon studied. In other cases, this risk is the consequence of a calibration based on a numerically limited data-base or which does not allow an adequate number of clusters of assimilable episodes to be distinguished and therefore obliges one to refer to average data obtained on very different case histories, thus emptying the calibration parameter found of significance.

These are the critical points very often found in the models studied where, either with the aim of obtaining a user-friendly tool, variables are omitted that are decisive in describing one of the three aspects conditioning the cost, or due to the limited sample, one is forced to assume calibration parameters derived as the average value of very different services, especially in terms of product and operating context.

Among the aspects of the LPT service that deserve greater attention in cost models is the product, the characteristics of which are expressed by the SOP. In fact, the latter, given the same quantity produced, i.e. the total mileage travelled, significantly influences the hourly commitment of drivers and rolling stock, and therefore has a far from negligible impact on the overall unit cost of production, given that drivers alone account for at least 50% of this. The characteristics of the SOP that affect the cost of drivers and rolling stock are various; among these, in particular, are the extension of the lines travelled and the frequency of the rides supplied on the same line, which together affect the downtime imposed on drivers and vehicles between one ride and the next, determining the annual mileage produced by these production factors and therefore the incidence of the relative cost item on the total.

The analysis developed in this study showed that many models cannot offer a sufficiently reliable prediction of cost because they are based on variables that do not sufficiently describe the characteristics of the SOP. Other methods, on the other hand, partially overcome this limitation by including variables such as driver performance, frequency of rides, line density and service time, or by incorporating dummy variables that describe certain characteristics of the SOP discontinuously or through synthetic parameters such as drivers' driving hours.

Advances in standard cost estimation research toward a greater interpretative capacity of the models can achieved by operating in two directions. The first is the built of new models that also include variables capable of describing in sufficient detail the product, i.e. the SOP. The second consists in the development of tools to obtain more reliable results from those existing models that also comprise efficiency variables indirectly linked to the SOP characteristics. These tools must materialize in relationships between the characteristics of the SOP and the representative parameters of efficiency included in these cost models. Through these relationships we will be able to determine the values ​​of the parameters in question with reference to the specific product rather than as an average of a sample including products necessarily different from each other.

References

- Official Journal of the European Union, Directive (EU) no. 2007/1370 of the European Commission, modified with Directive (EU) no. 2338/2016.

- Petruccelli, U.; Carleo, S. Cost models for local road transit. Public Transp 2017, 9, 527-548. DOI 10.1007/s12469-017-0162-2.

- Della Lucia, L.; Meneguzzer, C.; Zottis, G. Un approccio semplificato al calcolo del costo standard nel Trasporto Pubblico Locale - A simplified approach to the determination of standard cost in Local Public Transport. IF 2018, 11, 883-898.

- Petruccelli, U.; Racina, A. Stima del numero di conducenti come strumento di efficientamento dei servizi di trasporto pubblico - Assessment of the number of drivers as a tool for improving the efficiency of public transit services. IF 2019, 4, 295-315.

- Avenali, A.; Boitani, A.; Catalano, G.; D’Alfonso, T.; Matteucci, G. Un modello per la determinazione del costo standard nei servizi di trasporto pubblico locale su autobus in Italia. Econ Polit Ind 2014, 4, 181-213.

- Petruccelli, U.; Vuono, P. Il fondo nazionale e le politiche del trasporto pubblico locale tra obiettivi di efficienza ed equità metodo per la stima del costo del trasporto pubblico su gomma - The National Fund and local public transport policies between efficiency and equity objectives. IF 2024, 2, 103-130. Available online: https://www.medra.org/servlet/view?lang=it&doi=10.57597/IF.02.2024.ART.2)

- Petruccelli, U. Indicazioni per l’aggiornamento del D.M. sui costi standard per il trasporto pubblico locale. In Proceedings of the 4° Convegno, Sistema su Gomma nel Trasporto Passeggeri, Roma, Italia, Giornate di Studio 13 e 14 Ottobre 2021.

- Simonelli, F.; Papola, A.; Tinessa, F.; Tocchi, D.; Marzano, V. Towards a bottom-up estimation of a standard unit operating cost for bus operators: Methodology and policy implications in Italy. Transp. Policy 2023. Available online:  https://doi.org/10.1016/j.cstp.2023.101017

- Filippini, M.; Maggi, R.; Prioni, P. Cost-based Yardstick Regulation in the Swiss Regional Public Bus Industry. 1st Swiss Transport Research Conference, Monte Verità/Ascona, 2001. Available online:  https://www.researchgate.net/publication/228603705_Cost-based_Yardstick_Regulation_in_the_Swiss_Regional_Public_Bus_Industry(accessed on 3 March   2016).

- Fraquelli, G.; Piacenza, M.; Abrate, G. Regulating Public Transit Networks: How do Urban Intercity Diversification and Speed up Measures Affect Firms’ Cost Performance?. Annals of Public and Cooperative Economics 2004, 75(2), 193-225.

- Cirianni, F.; Leonardi, G.; Iannò, D. Model definition and optimization of unit cost functions in service integration of bus services. In Proceedings of the 6th International Symposium “Networks for Mobility”, 27/28 Settembre 2012, 1-11.

- ANAV, La Sapienza University. La determinazione del costo standard nei servizi di trasporto pubblico locale su autobus: aspetti metodologici e prime esperienze applicative. Residenza di Ripetta, Roma, 2013. Available online: http://www.filt.veneto.cgil.it/sites/default/files/2014%201%2017%20CSOPTI%20STANDARD%20TPL.pdf (accessed on 18 December 2015).

- ASSTRA. Un modello di calcolo del costo standard per il trasporto pubblico locale e regionale automobilistico. Audizione presso la IX Commissione trasporti, poste e telecomunicazioni della Camera dei Deputati, Roma, 2013. Available online:  https://www.camera.it/temiap/temi17/Asstra%20%20percorsi%20di%20sviluppo%20per%20il%20trasporto%20pubblico%20locale.pdf.

- Di Gangi, M. Un approccio analitico per la valutazione dei costi nei sistemi di trasporto collettivo.  I sistemi di trasporto nell’area del Mediterraneo: infrastrutture e competitività; Bergantino, A.S.; Carlucci, F.; Cirà, A.; Marcucci, E.; Musso, E. Franco Angeli: Milano, 2013, 347-358.

- Regione Veneto. Costruzione del Costo Standard 2010 per il TPL su gomma e metrotram” - Descrizione del metodo, delle attività svolte e delle risultanze raggiunte relazione finale. 12 marzo 2013, allegato G alla D.gr n. 686 del 14 maggio 2013;

- Regione Friuli-Venezia Giulia. Piano Regionale del Trasporto Pubblico Locale -  Relazione Illustrativa. 4 Aprile 2013, delibera n.605.

- Avenali, A.; Boitani, A.; Catalano, G.; D’Alfonso, T.; Matteucci, G. Assessing standard costs in local public bus transport: A hybrid cost model. Transp. Policy 2018, 62, 48-57.

- Dalen, D. M.; Gomez Lobo, A. Regulatory contracts and cost efficiency in the Norwegian Bus Industry: Do high-powered contracts really work. 7th Annual Conference of the Latin American and Carribean Economic Association, Discussion Paper 6, 2002. Available online:  http://brage.bibsys.no/xmlui/bitstream/handle/11250/94048/1/Dalen1dp2002-06.pdf. (accessed on 11 February 2016).

- Cubukcu, K. Mert. Cost of Urban Bus Transit Operations and Geography of Service Territory. Conference of Regional and Urban Modeling, Free University of Brussels, 2006. Available online: http://ecomod.net/sites/default/files/document-conference/ecomod2006-rum/1149.pdf  (accessed on 20 January 2016).

- Abrate, G.; Erbetta, F.; Fraquelli, G.; Vannoni, D. Cost function estimation of multi-service firms. Evidence from the passenger transport industry. Carlo Alberto Notebooks, n. 380, 2014. Available online: http://www.carloalberto.org/assets/working-papers/no.380.pdf.

- Avenali, A.; Boitani, A.; Catalano, G.; D’Alfonso, T.; Matteucci, G. Assessing standard costs in local public bus transport: evidence from Italy. Transp. Policy 2016, 52, 164-174.

- Official Gazette of the Italian Republic, Ministry of Infrastructure and Transport, Decree 28/03/2018 no.157 (Definizione dei costi standard dei servizi di trasporto pubblico locale e regionale e dei criteri di aggiornamento e applicazione).

- Petruccelli, U.; Ciampa, S.; Diomedi, M.; Olita, S. L’offerta di trasporto pubblico locale in Italia: analisi dei dati dell’Osservatorio Nazionale per le Politiche del Trasporto Pubblico Locale - Local public transport supply in Italy: analysis of data from the National Observatory for Local Public Transport Policies. IF 2020, 9, 599-632.

- Cambini, C.; Piacenza, M.; Vannoni, D. Restructuring Public Transit Systems: Evidence on Cost Properties from Medium and Large-Sized Companies. Rev Ind. Organ. 2007, 31, 183–203.

- Pulley, L.B.; Braunstein, Y.M. A Composite Cost Function for Multiproduct firms with an application to Economies of Scope in Banking. Rev Econ Stat 1992, 74, 221-230.

- Petruccelli, U.; Racina, A. Feeder-trunk and direct-link schemes for public transit: a model to evaluate the produced accessibility. Public Transp 2021, 13, 301–323. Available online:  https://doi.org/10.1007/s12469-021-00262-4.

University of Basilicata - (email: umberto.petruccelli@unibas.it)

University of Basilicata